IO CARD FIRMWARE

Technical Review Document

Document Rev	2.0

Date	March 2026

Author	Rohit Jadhav

Target	STM32F4  —  CAN 1 Mbps

Hex	IO-Card-Rohit.hex

Scope	main.c  /  engine.c  /  operations.c

| 2 Critical Fixed | 2 Medium Fixed | 3 Low Fixed | 1 Open Medium | 4 Open Low | 14 Future Items |
| --- | --- | --- | --- | --- | --- |

01  SCOPE & PURPOSE

This document reviews the current IO card firmware (March 2026, Rohit Jadhav) against the previous release baseline (February 2026) and the legacy codebase (2021, Chetan). It records all changes verified, issues resolved, issues outstanding, and items identified for future development cycles.

The firmware drives a 64-channel digital IO expansion card over CAN bus at 1 Mbps, implemented on an STM32F4 microcontroller. Three source files are in scope:

- main.c  —  peripheral init, CAN config, main loop, RX callback

- engine.c  —  GPIO control, MUX line selection, ADC, card ID detection

- operations.c  —  CAN command dispatch layer

02  CONFIGURATION PARAMETERS

All hardware configuration values compared across legacy and current versions. Highlighted rows indicate changed values.

CAN Bus  (MX_CAN1_Init)

| Parameter | Legacy (2021) | Current (2026) | Δ |
| --- | --- | --- | --- |

| Instance | CAN1 | CAN1 | — |
| --- | --- | --- | --- |

| Prescaler | 3 | 5 | ▲ |
| --- | --- | --- | --- |

| Mode | NORMAL | NORMAL | — |
| --- | --- | --- | --- |

| SyncJumpWidth | SJW_1TQ | SJW_1TQ | — |
| --- | --- | --- | --- |

| TimeSeg1 (BS1) | BS1_11TQ | BS1_6TQ | ▲ |
| --- | --- | --- | --- |

| TimeSeg2 (BS2) | BS2_3TQ | BS2_2TQ | ▲ |
| --- | --- | --- | --- |

| AutoRetransmission | DISABLE | DISABLE | — |
| --- | --- | --- | --- |

| Resulting bit-rate | 1 Mbps | 1 Mbps | — |
| --- | --- | --- | --- |

CAN TX Header  (txHeader)

| Parameter | Legacy (2021) | Current (2026) | Δ |
| --- | --- | --- | --- |

| DLC | 8 | 8 | — |
| --- | --- | --- | --- |

| IDE | STD | STD | — |
| --- | --- | --- | --- |

| RTR | DATA | DATA | — |
| --- | --- | --- | --- |

| StdId | 0x030 | 0x200 + Card[3] | ▲ |
| --- | --- | --- | --- |

| ExtId | 0x02 | 0x02 | — |
| --- | --- | --- | --- |

CAN Filter  (canfil)

| Parameter | Legacy (2021) | Current (2026) | Δ |
| --- | --- | --- | --- |

| FilterBank | 0 | 0 | — |
| --- | --- | --- | --- |

| FilterMode | IDMASK | IDMASK | — |
| --- | --- | --- | --- |

| FilterFIFO | RX_FIFO0 | RX_FIFO0 | — |
| --- | --- | --- | --- |

| FilterIdHigh | (0x7D0+Card[3])<<5 | (0x7D0+Card[3])<<5 | — |
| --- | --- | --- | --- |

| FilterMaskIdHigh | 0xFFFF | 0xFFFF | — |
| --- | --- | --- | --- |

| FilterScale | 32BIT | 32BIT | — |
| --- | --- | --- | --- |

| SlaveStartFilterBank | 14 | 14 | — |
| --- | --- | --- | --- |

ADC2  (MX_ADC2_Init)

| Parameter | Legacy (2021) | Current (2026) | Δ |
| --- | --- | --- | --- |

| Instance | ADC2 | ADC2 | — |
| --- | --- | --- | --- |

| ClockPrescaler | PCLK_DIV4 | PCLK_DIV4 | — |
| --- | --- | --- | --- |

| Resolution | 12-bit | 12-bit | — |
| --- | --- | --- | --- |

| NbrOfConversion | 2 | 2 | — |
| --- | --- | --- | --- |

| Ch0 SamplingTime | 15 cycles | 15 cycles | — |
| --- | --- | --- | --- |

| Ch3 SamplingTime | (inherits) | (inherits) | — |
| --- | --- | --- | --- |

| ADC_ch3() override | 480 cycles | 480 cycles | — |
| --- | --- | --- | --- |

USART1  (MX_USART1_UART_Init)

| Parameter | Legacy (2021) | Current (2026) | Δ |
| --- | --- | --- | --- |

| BaudRate | 115200 | 115200 | — |
| --- | --- | --- | --- |

| WordLength | 8-bit | 8-bit | — |
| --- | --- | --- | --- |

| StopBits | 1 | 1 | — |
| --- | --- | --- | --- |

| Parity | None | None | — |
| --- | --- | --- | --- |

| Mode | TX_RX | TX_RX | — |
| --- | --- | --- | --- |

System Clock  (SystemClock_Config)

| Parameter | Legacy (2021) | Current (2026) | Δ |
| --- | --- | --- | --- |

| HSE | ON | ON | — |
| --- | --- | --- | --- |

| PLLM | 8 | 8 | — |
| --- | --- | --- | --- |

| PLLN | 180 | 180 | — |
| --- | --- | --- | --- |

| PLLP | DIV2 | DIV2 | — |
| --- | --- | --- | --- |

| AHB Divider | DIV1 | DIV1 | — |
| --- | --- | --- | --- |

| APB1 Divider | DIV4 | DIV4 | — |
| --- | --- | --- | --- |

| APB2 Divider | DIV2 | DIV2 | — |
| --- | --- | --- | --- |

| SYSCLK | 180 MHz | 180 MHz | — |
| --- | --- | --- | --- |

| APB1 | 45 MHz | 45 MHz | — |
| --- | --- | --- | --- |

| Flash Latency | 5 | 5 | — |
| --- | --- | --- | --- |

Note on CAN bit-rate: both legacy and current configurations produce 1 Mbps. Legacy used prescaler 3 with BS1=11TQ / BS2=3TQ (15 TQ total). Current uses prescaler 5 with BS1=6TQ / BS2=2TQ (9 TQ total). At APB1=45 MHz both resolve to 45,000,000 ÷ prescaler ÷ total_TQ = 1,000,000 bps.

03  RESOLVED ISSUES

The following issues identified in the previous review have been confirmed resolved in the current codebase.

FIX-01 Error_Handler — unsafe CAN IRQ call removed RESOLVED File main.c → Error_Handler() Previous Called HAL_CAN_IRQHandler(&hcan1) after __disable_irq() — undefined behaviour Current while(1); — safe halt. IRQs disabled, no further HAL calls. Severity Critical

FIX-02 CAN TX address hardcoded — replaced with dynamic card ID RESOLVED File main.c → txHeader.StdId Previous 0x030 (fixed — host cannot distinguish which card responded) Current 0x200 + Card[3] (e.g. card 4 → 0x204). Enables multi-card addressing. Severity Critical

FIX-03 CAN restart path missing notification activation RESOLVED File main.c → CAN start block (else branch) Previous HAL_CAN_Start() retry called but HAL_CAN_ActivateNotification() never called — RX IRQ would never fire Current HAL_CAN_ActivateNotification() added to both success and retry paths Severity Medium

FIX-04 Control_line() bulk ops wrote all pins to wrong port RESOLVED File engine.c → Control_line() cases 0 and 65 Previous All 64 pins written to GPIOE regardless of actual port — bulk reset and bulk set were non-functional Current Each pin now addressed on its correct port (GPIOA/B/C/D/E/F/G per schematic). Comment added: 'Set all control pins LOW/HIGH on their respective ports' Severity Critical

FIX-05 result[] array size mismatch across translation units RESOLVED File engine.c vs operations.c Previous engine.c declared result[8], operations.c declared extern result[9] — interface mismatch Current engine.c now declares result[9] — sizes consistent. Index 8 allocated but not written; DLC=8 so only bytes 0–7 transmitted. Severity Medium

FIX-06 single_write() TX opcode used variable instead of literal RESOLVED File operations.c → single_write() Previous csend[] = {operation, pin_no, ...} — if called outside normal dispatch, wrong opcode sent Current csend[] = {0x02, pin_no, ...} — opcode hardcoded, consistent with all other operations Severity Low

FIX-07 Unused global variables removed RESOLVED File main.c → global scope Previous Variables j (uint8_t), x (uint32_t), c (unsigned char) declared and never used Current All three removed Severity Low

FIX-08 Boot delay reduced RESOLVED File main.c → post GPIO-init delay Previous HAL_Delay(1000) — 1 second Current HAL_Delay(500) — 500 ms Severity Low

FIX-09 Raw GPIO pin numbers replaced with named macros in main loop RESOLVED File main.c → switch(operation) cases Previous GPIO_PIN_12 and GPIO_PIN_13 used directly Current C_Pin and D_Pin macros used throughout Severity Low

04  OPEN ISSUES

The following issues are confirmed present in the current codebase and have not yet been addressed.

OPEN-01 CAN RX input validation not yet implemented OPEN — MEDIUM File main.c → HAL_CAN_RxFifo0MsgPendingCallback Detail canRX[0..3] are assigned directly to globals with no range checking. Any byte value 0–255 is accepted. operation should be 1–5, pin_no should be 1–64, pin_status should be 0 or 1. Risk A malformed or unexpected CAN frame can trigger wrong operations or drive incorrect GPIO states. Fix Add range guards in the callback before assigning to globals. See Section 05 item FD-01 for recommended implementation.

OPEN-02 Card_select() always returns 0 OPEN — LOW File engine.c → Card_select() Detail counter1 is reset to 0 before the return statement. Card[3] is correctly set before the reset so all callers work correctly, but the return value is always 0 and is therefore meaningless. Fix Return Card[3] directly, or change return type to void.

OPEN-03 ADC_read() uses raw GPIO_PIN_12/13 instead of named macros OPEN — LOW File engine.c → ADC_read() Detail GPIO_PIN_12 and GPIO_PIN_13 used directly. Inconsistent with main.c which uses C_Pin and D_Pin. Fix Replace with C_Pin / D_Pin macros. Best addressed during next ADC cleanup pass.

OPEN-04 select_status variables declared at file scope OPEN — LOW File engine.c → global scope Detail select_status1 through select_status6 are used only inside Select_line(). Declaring them globally wastes 24 bytes of RAM and risks accidental reuse. Fix Move declarations inside Select_line() as local variables.

OPEN-05 UART initialised but never used OPEN — INFO File main.c → MX_USART1_UART_Init() Detail USART1 configured at 115200 baud but no code transmits or receives. The peripheral is ready to use. Fix No action required. Consider enabling for debug output — particularly useful alongside Error_Handler improvements.

05  FUTURE DEVELOPMENT

The following items are not current bugs but are identified as improvements for future development cycles. Grouped by priority.

5.1  High Priority

FD-01 CAN RX input validation HIGH All fields from the CAN frame should be range-checked before being accepted. Recommended implementation: if(op < 1 ¦¦ op > 5) return; // operation 6 (analog) removed if(pin < 1 ¦¦ pin > 64) return; // pin out of range if(sts != 0 && sts != 1) return; // invalid state Use local temporaries (op, pin, dir, sts) before assigning to globals to minimise the window during which globals are in a partial state.

FD-02 ISR-shared globals must be declared volatile HIGH operation, pin_no, pin_dir, pin_status, receive_flag are written in the CAN RX ISR and read in the main loop. Without volatile the compiler may legally cache or reorder these accesses. Possible symptoms: missed commands, stale values, correct behaviour in Debug but failures in Release builds. Fix: declare all five as volatile — e.g. volatile unsigned int operation, pin_no, pin_dir, pin_status; and volatile int receive_flag;

FD-03 Race condition — no atomic copy between ISR and main loop HIGH A second CAN frame arriving while the main loop is processing the first will overwrite the shared globals mid-execution. There is no queue, shadow copy, or critical section. Simplest fix: at the start of main loop processing, copy all command fields atomically under a brief interrupt-disable window, then release interrupts before executing the command. Robust fix: implement a small ring buffer for pending commands so no frame is lost under burst traffic at 1 Mbps.

FD-04 CAN transmit return status ignored everywhere HIGH Every HAL_CAN_AddTxMessage() call in operations.c discards the return value. If the mailbox is full, bus-off, or CAN was not fully started, the response fails silently. Fix: check return value and set an error flag, retry, or at minimum report over UART. Silent failure is very difficult to diagnose in production.

5.2  Medium Priority

FD-05 Error_Handler — no recovery strategy MEDIUM Any HAL error permanently halts the firmware with interrupts disabled. One peripheral fault freezes the card until power cycle. Recommended path: configure IWDG watchdog so a hung device resets automatically. Store a fault code in a backup register before halting so the cause is visible after reset. Optionally transmit a fault frame over CAN/UART before halting.

FD-06 Header include case mismatch — Engine.h vs engine.h MEDIUM operations.c and engine.c include "Engine.h" (capital E). The file in the project tree is engine.h (lowercase). On case-sensitive filesystems (Linux / GCC ARM toolchain) this will fail to compile. Fix: standardise all includes to engine.h and rename the header file accordingly.

FD-07 pin_dir received but never used MEDIUM The RX callback stores pin_dir from canRX[2] but no function reads it to control GPIO direction or routing. This is either an incomplete protocol feature or a stale field. Action required: clarify with the OS/host protocol specification and either implement the direction logic or remove the field from the protocol to avoid confusion.

FD-08 single_read() drives Control_line before reading — verify intent MEDIUM single_read() calls Control_line(pin_no) then Select_line(pin_no-1) then Read_pin(). This actively drives the control output on the line being read. If passive input sensing is intended, the Control_line() call should be removed or the line driven to a known safe state before reading. Verify against the electrical schematic.

FD-09 pin_no underflow risk in Select_line(pin_no - 1) MEDIUM pin_no is unsigned int. If it were 0, subtracting 1 wraps to 0xFFFFFFFF and Select_line() decodes meaningless bitmask values into the mux control lines. Currently unguarded — CAN RX validation (FD-01) has not yet been implemented. A guard inside single_read() provides defence in depth regardless of the callback state.

5.3  Low Priority  /  Housekeeping

FD-10 ADC init configuration does not match runtime usage LOW MX_ADC2_Init() configures a 2-channel scan (CH0 rank 1, CH3 rank 2). ADC_read() reconfigures only CH3 as rank 1 via ADC_ch3() and reads one conversion. CH0 code is commented out. Since analog reads are not being used, the ADC init and all ADC-related functions (ADC_read, ADC_ch0, ADC_ch3, related globals) should be removed in the next cleanup pass.

FD-11 DLC=8 with shorter local response buffers LOW txHeader.DLC is set to 8 globally. Commands that use all 8 bytes are correct. Commands that use only 4-byte local arrays (single_read, single_write, reinit) transmit the remaining bytes from uninitialised stack space. Fix: zero-pad all local csend[] arrays to 8 bytes — e.g. uint8_t csend[8] = {0x01,0x01,0x00,0x00,0x00,0x00,0x00,0x00}; This ensures deterministic payload content without changing DLC.

FD-12 ADC_ch0() defined but never called LOW ADC_ch0() is defined in main.c. ADC_read() only calls ADC_ch3(). Remove when cleaning up ADC code.

FD-13 select_status1-6 should be local to Select_line() LOW Six unsigned int variables used only inside Select_line() are at file scope. Moving them local reduces global RAM by 24 bytes.

FD-14 UART ready for debug output INFO USART1 is configured at 115200 baud and ready to use. Adding fault reporting here — especially from Error_Handler and CAN TX failures — would significantly aid field diagnostics.

06  CHANGE LOG

Summary of all changes from legacy baseline (2021) through current version (March 2026).

main.c

- recive_flag  →  receive_flag  —  typo corrected

- HAL_Delay(1000)  →  HAL_Delay(500)  —  faster boot

- txHeader.StdId  changed from fixed 0x030 to dynamic 0x200 + Card[3]

- CAN restart else-branch  —  HAL_CAN_ActivateNotification() added

- Error_Handler  —  HAL_CAN_IRQHandler() call removed; while(1) safe halt added

- GPIO_PIN_12 / GPIO_PIN_13  →  C_Pin / D_Pin  in main loop

- Unused globals removed  —  j, x, c

- CAN RX validation  —  not yet implemented (open item OPEN-01)

engine.c

- Control_line() case 0  —  all 64 pins now reset on correct ports (was all GPIOE)

- Control_line() case 65  —  all 64 pins now set on correct ports (was all GPIOE)

- result[8]  →  result[9]  —  size matches extern in operations.c

operations.c

- single_write()  —  opcode byte changed from {operation,...} to {0x02,...}

- Opcode comments added to multiple_read() (0x03) and card_status() (0x04)

IO Card Firmware  —  Technical Review Document  Rev 2  —  March 2026  —  Rohit Jadhav